FOCAL LENGTH and FORMAT

Focal length relates to Format. So we can imagine that if our film stock size increases we must increase our focal length.

16 mm film needs about a 25 mm focal length to produce the same image size as 35mm film with a 50 mm focal length.

With digital video the same holds true. 2/3” imager CCD on professional digital cameras needs a longer focal length lens than the ½” CCD on most pro-sumer digi-cameras to produce the same image size (similar angle of view)

| Format/Frame Size | Normal | Wide | Ultra-Wide (fish-eye) | Telephoto |
| --- | --- | --- | --- | --- |
| 8mm Film | 12.5 mm |  |  | 17.5mm or More |
| 16mm Film | 17.5-25mm | 12mm-16mm |  | 40mm or More |
| 35mm Film | 35-50mm | 24mm-34mm | 6mm (widest ever-220degree field of view) 8mm-10mm(circular) 15mm-16mm (full frame) | 70mm or More |
| 70mm Film | 100mm | 48mm-68mm | 20-30mm | 140mm or More |
| .25 In. CCD Video Chip (GL1) | 12mm |  |  |  |
| .33 In. CCD Video Chip (Sony HDV, Canon XL1 and XL2, Panasonic AG-HVX200 | 6.8-9.7mm | 3-5mm | 2.2-2.8mm | 19.5 mm or more |
| .66 in. CCD (Digi beta) | 12.3mm-17.6mm | 5.5mm-7.8mm | 4mm-5mm | 30mm or more |

| Comparison of CCD Specifications of some 3-CCD DV Camcorders |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| CAMERA | CCD size |  | pixel count |  | pixel shift | DSP |
|  |  |  | total | effective |  |  |
| Canon XL2 |  | 1/3 | 680,000 | 460,800* | horizontal | 12-bit |
| Canon XL1 / XL1S |  | 1/3 | 270,000 | 250,000 | horiz & vert | 8-bit |
| Panasonic DVX100A |  | 1/3 | 410,000 | 380,000 | horizontal | 12-bit |
| Sony VX2100 |  | 1/3 | 380,000 | 340,000 | N/A | ? |
| Canon GL2 |  | 1/4 | 410,000 | 380,000 | horiz & vert | 8-bit |
| Panasonic DVC30 |  | 1/4 | 410,000 | 380,000 | horizontal | 12-bit |
| Sony TRV950 |  | 1/4.7 | 1,070,000 | 690,000 | N/A | 14-bit |
| Panasonic PVGS200 |  | 1/6 | 800,000 | 440,000 | N/A | ? |
| * XL2 in 16:9 mode only (345,600 effective pixels in 4:3 mode) |  |  |  |  |  |  |

| Focal Length Comparison |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 1/3in (.33) | 1/2in (.5) | 2/3in (.66) | S16mm | 35mm | Angle of View |
| 2.2 mm | 3.1 mm | 4 mm | 5.7 mm | 12 mm | 95° |
| 2.5 mm | 3.5 mm | 4.5 mm | 6 mm | 13 mm | 89° |
| 2.8 mm | 3.9 mm | 5 mm | 7 mm | 14 mm | 82° |
| 3 mm | 4.25 mm | 5.5 mm | 8 mm | 16 mm | 77° |
| 4.3 mm | 6 mm | 7.8 mm | 11 mm | 22 mm | 59° |
| 6.8 mm | 9.5 mm | 12.3 mm | 17.5 mm | 35 mm | 41° |
| 9.7 mm | 13.6 mm | 17.6 mm | 25 mm | 50 mm | 28° |
| 16.5 mm | 23 mm | 30 mm | 40 mm | 85 mm | 18° |
| 19.3 mm | 27 mm | 35 mm | 50 mm | 100 mm | 15° |
| 39 mm | 54 mm | 70 mm | 100 mm | 200 mm | 7° |
| 45 mm | 63.5 mm | 82 mm | 120 mm | 240 mm | 6° |
| 86 mm | 121 mm | 156 mm | 200 mm | 400 mm | 3.25° |
| 172 mm | 242 mm | 312 mm | 400 mm | 800 mm | 1.75° |